ABSTRACT

IP Sentinel is a IoT-based wireless monitoring and Access Control System developed using the ESP8266 microcontroller. The system operates in Soft Access Point mode, allowing devices to connect directly to it while automatically assigning IP addresses through its internal DHCP server. Each connected  device is identified using its MAC address and verified against a predefined authorization list. This enables the system to distinguish between approved and unauthorized devices in real time.

When an unknown device connects, the system immediately activates visual and audible alerts to signal potential unauthorized access. In addition, an embedded web dashboard with administrator authentication provides real-time monitoring, device management, and connection time tracking. By integrating wireless networking, MAC-based authorization, and an embedded web interface, IP Sentinel demonstrates a practical and cost-effective approach to small-scale network security monitoring using IoT technology.

SYSTEM ARCHITECTURE

Block Diagram:

ESP8266 → Wi-Fi AP → Web Dashboard → LEDs/Buzzer.

Hardware Requirements:

| Component | Description |
| --- | --- |
| ESP8266 | Microcontroller with Wi-Fi AP mode |
| Green LED | Indicates authorized devices |
| Red LED | Indicates unauthorized devices |
| Active Buzzer | Audio alert for unauthorized access |
| Breadboard | Circuit assembly platform |
| Jumper Wires | Connections between components |
| USB Cable | Power and programming interface |

Software Requirements:

| Software/Library | Purpose |
| --- | --- |
| Arduino IDE | Programming and uploading code |
| ESP8266WiFi library | Enables Wi-Fi and AP functionality |
| ESP8266WebServer library | Hosts the local web dashboard |
| Web Browser | Accesses the dashboard via IP |

IMPLEMENTATION

Circuit Diagram:

The circuit connects the ESP8266 microcontroller to two LEDs and a buzzer. The green LED is connected to pin D1 (GPIO 5), the red LED to D3 (GPIO 0), and the buzzer to D2 (GPIO 4). Each component is grounded through separate GND pins of the ESP8266.

The green LED indicates that all connected devices are authorized. The red LED and buzzer activate when unauthorized devices are detected, providing both visual and audio alerts.

Screenshots:

.

RESULTS

System Working Status:

Successfully detects connected devices, distinguishes authorized vs unauthorized.

Output Results:

| Device Status | Green LED | Red LED | Buzzer | Dashboard Display |
| --- | --- | --- | --- | --- |
| Authorized | ON | OFF | OFF | Green “AUTHORIZED” |
| Unauthorized | OFF | ON | ON | Red “UNAUTHORIZED” |